# Feature description of the generic PlugIn Prototype V1

This document describes the generic PlugIn developed for the IS&V Project. This is a Prototype of a so called Black-Box to control the behavior of the machine tool. First an architecture overview is given followed by a detailed description of the features, which are supported by this generic PlugIn.

## Miscellaneous

### How does the PlugIn work with the UG environment:

The VNC (Virtual NC) in the main part of IS&V to configure and simulate the behavior of the machine tool controller. One mode to configure the VNC is the PlugIn mode. In this mode an external library, a Plugin, in form of a DLL or external program, is used in order to parse and interpret the NC part program and control and drive behavior of the machine tool. With such kind of module it is possible to have a close to reality solution as it bests.

This generic PlugIn module that is currently used in the CAM setup context, can be in productive release also used to interpret and simulate a stand-alone G codes program outside the CAM setup context.

### UG related items

The PlugIn machine tool driver has only three event handlers.

MOM_SIM_initialize_mtd

MOM_SIM_exit_mtd

MOM_SIM_execute_nc_command.

MOM_SIM_initialize_mtd is invoked before simulation starts and there the controller mode (Plugin mode) has to be set and the Plugin has to be initialized with a call to the function SIM_PLUGIN_initialize. At each point where NC program line is generated, the SIM_execute_nc_command event is triggered which invokes the MOM_SIM_exceute_nc_command handler. This handler passes the NC line to the Plugin to parse and execute it. When simulation ends, the SIM_exit_mtd event is triggered for proper cleanup and exit from the Plugin. From the MOM_SIM_exit_mtd handler, the function SIM_PLUGIN_exit is called for proper exit from Plugin.

The functions from Plugin are invoked from the TCL machine tool driver via the MOM_run_user_function (e.g. MOM_run_user_function "$plugin_library" SIM_PLUGIN_initialize).

#### SIM_PLUGIN_initialize

This procedure is executed before the simulation starts. It supposes to initialize the machine tool driver. First, it sets the controller mode to Plugin mode. Then the path of the DLL had to be set.

#### SIM_PLUGIN_exit

This procedure is for exiting the machine tool driver. This procedure is called when the machine tool simulation ends. A cleanup in all loaded modules can be done.

#### SIM_PLUGIN_execute_nc_command

This procedure executes the next NC command. It is invoked only in Plugin mode. It invokes the controller Plugin in order to process the next command. The NC command line to be processed is stored in the variable mom_sim_nc_command and can be access inside the Plugin via a user function. This function calls a special TCL procedure, which parses and interprets the NC line. Based on the information found in the NC line, the related SV user functions are called to create the SV events, which drives the machine tool simulation.

### DLL related items for the example of the Plug-In

As discussed dCADE will use the know-how and also as much code as possible of the existing controller technology of the UNISIM controller. Most of this code is available in TCL. To follow the Plugin DLL concept it is possible to fill the functionality of the 3 functions with what ever we need to run our TCL code. Therefore inside the Generic Plugin DLL a separate TCL controller is created and initialized; totally independent from UG MOM TCL. This new interpreter used the existing code of the controller. So the Plugin DLL will use the user function library and its responding headers only. Thus we are out of the developing environment of UG like costumers or partners will do. So we can offer a clean API to configure the PlugIn and drive the machine tool simulation.

### Conceptual architecture of the generic PlugIn Prototype

Figure:	Simple design structure of the generic PlugIn solution

## Installation and settings

### Configure the MTD

To use inside the generic PlugIn DLL a separate TCL interpreter it is necessary to have some TCL initialization data available. To inform the DLL about the location of the data some TCL folders has to be copied. Furthermore a global environment variable has to be set for the initialization, which points to this directory.

ENVIRONMENT VARIABLE:

Name: PLUGIN_TCL_LIBRARY

Value: The path to the ..\lib\tcl8.3 directory

Example: A part of an example of a machine tool driver is shown as follows:

This can be found in the isv_kit directory (e.g. ..MACH\auxiliary\machine_simulation\machine_tool_driver)

This code is taken out of the mtd_mill3ax_plugin.tcl MTD

This is called to allow the MTD access to generate the normal NC output part program based on the generic MOM Post as it is used normally in UG without changing anything on the POST.

#______________________________________________________________________________

#The following command invokes the corresponding event handler to generate the

#NC program

#______________________________________________________________________________

source [MOM_ask_env_var UGII_CAM_POST_DIR]mill3ax.tcl

This code is to configure the configurable user interface for machine tool simulation in the ISV. This is the default settings and shows most of the necessary data during simulation.

# Define default system dialog table

set dialog_num_items 0

set mom_sim_dialog_attributes  "TITLE=<Simulation Control Panel>"

…

set mom_sim_dialog_table(9)   "SYS_MACHINE_TIME_ITEM   S10"

set mom_sim_dialog_table(10)  "SYS_LAST_ITEM"

Here the above described TCL Variable can be set. In the example this is already done in a start batch script, so this it not longer necessary here.

# The environment variable for the lib path of the TCL

# Set in the start batch

# set env(PLUGIN_TCL_LIBRARY) "E:/ugs/Development/PlugIn/lib"

The following shows one of the three handlers described above. This handler is for initialization and shows how the initialization procedure of the PlugIn DLL in called.

proc MOM_SIM_initialize_mtd {} {

#_______________________________________________________________________________

# This procedure is executed before the simulation starts. it suppose to initialize

# the machine tool driver. First, it sets the controller mode and in case of Plugin

# mode, it also sets the path and DLL for the PLUGIN and call the controller

# plugin initialization

#_______________________________________________________________________________

#

# initializing MTD and setting controller mode.

SIM_mtd_init NC_CONTROLLER_PLUGIN

# define the path of the Plugin DLL

set plugin_library "E:\\ugs\\Development\\PlugIn\\Plugin_dlls\\generic.dll"

# Invoke plugin initialization function in plugin dll

MOM_run_user_function "$plugin_library" SIM_PLUGIN_initialize

}

### Initialization data

For the initialization the Plugin needs some information from the machine tool and the controller. The following list gives an overview of the needed information. Due to the fact that we will also simulate external G codes the source for the information should be MOM independent. This list is given for the example of the generic Plugin. In later versions it should be possible to get as much information as possible out of the machine tool model (KIN) and out of the CAM setup. For later purpose it should be not necessary to have a MOM Post and a CAM part with Operation for simulate external M&G code, but IS&V still relay on a CAM setup in every case.

PLUGIN_TCL_LIBRARY

Description:	Environment variable for the lib path of TCL.

Example:	set env(PLUGIN_TCL_LIBRARY) “E:\ugs\mach\plugin\lib”

PLUGIN_DEBUG

Description:	Set variable to activate DEBUG output inside the generic PlugIn DLL. All debug code will go to a UG output message window. If this variable is not set the system creates no debug output window.

Example:	set env(PLUGIN_DEBUG) "ON"

TCL Global Variables

Description:	The following is the list of global TCL variables that have to be initialized at initialization time before the first NC line is interpreted. Those variables are accessed by the Plugin to extract data needed for generating the proper S&V commands. Related to the fact, that the generic PlugIn is also doing the most work in  TCL this list of variables allows it easy to extend the data coming from MOM Post or UG and transfer it to the external TCL interpreter of the generic PlugIn without recompile the DLL. Only be adding the name of a variable to the list and set it afterwards. This transfers the data to the external TCL interpreter of the generic PlugIn.

Example:	This list an initial list for a generic plugin and is not yet complete.

mom_plugin_tooldata
mom_plugin_circip_mode 
mom_plugin_cording_tol
mom_plugin_kim_comp_for_toolmount
mom_plugin_junction_for_toolmount
mom_plugin_tool_length_correction
mom_plugin_axis_names
mom_plugin_zcs_component_name

mom_plugin_tooldata

Description:	The data of the tools are stored in one variable for each tool. Each parameter is described with a KEYWORD (KEYWORD=<value> KEYWORD=<value> …)

An additionally variable stores the number of tools.

mom_plugin_tooldata          ;# string array

mom_plugin_tooldata_count    ;#  number of array elements

Example:	set mom_plugin_tooldata(0) " TOOL_NUMBER=<03> LENGTH=<7.205> DIAMETER=<40> UG_NAME=<UGT0201_019>"

set mom_plugin_tooldata(1) " TOOL_NUMBER=<05> LENGTH=<13.268> DIAMETER=<27.7> UG_NAME=<UGT0303_007>"

set mom_plugin_tooldata_count 2

mom_plugin_circip_mode

Description:	Circular interpolation mode (1 absolute 2 incremental) Default is 1

Example:	set mom_plugin_circip_mode 2

mom_plugin_cording_tol

Description:	Tolerance for interpolation during circular interpolation. If this variable is not set as a default this value will inquired by calling UF_SIM_ask_cording_tol. This is the tolerance from the controller of the SIM object.

Example:	set mom_plugin_cording_tol 0.01

mom_plugin_kim_comp_for_toolmount - mom_plugin_junction_for_toolmount

Description:	Name of the KIM component and the junction for the tool mount. Default for KIM component name is SPINDLE. Default for junction name is JNCT_SPINDLE, if the variables are not set.

Example:	set mom_plugin_kim_comp_for_toolmount "SPINDLE"
set mom_plugin_junction_for_toolmount "JNCT_SPINDLE"

mom_plugin_tool_length_correction

Description:	Tool length correction in xy plane. Default is on

Example:	set mom_plugin_tool_length_correction "ON"

mom_plugin_axis_names

Description:	List for the axis names of the degree of freedoms in the KIM model. For default the axis names are identical with the axis names, which are used in the NC code

Example:	set mom_plugin_axis_names "{X linear1} {Y linear2} {Z linear3} {A table}"
set mom_plugin_axis_names "{X X} {Y Y} {Z Z} {C C} {B SP}"

mom_plugin_zcs_component_name

Description:	Name of the component, were the ZCS junction is assigned to. In most cases this component can be the work piece component itself.

ATTENTION: If the work piece is mounted on a rotary axis then the component must be the last component in the KIM tree, which not rotates. Otherwise the output in the UI is not correct

This variable is necessary to set the ZCS junction with the position of the msys related to the work piece geometry. Without this information the user interface can't display the correct values for the tool position and orientation.

Example:	set mom_plugin_zcs_component_name "WORKPIECE"

mom_plugin_mtcs_to_zcs_offset

Description:	Offset between the MTCS and the ZCS. At the moment the generic Plugin only supports simple offset no transformation.

This is not longer necessary. The data is extracted from the internal model by inquire data via user functions. ATTENTION It is assumed that a msys exists! This is issued by the % or # sign seen as the first NC line

Example:	set mom_plugin_mtcs_to_zcs_offset "0 0 0"

mom_plugin_rtcp_data

Description:	This data is only necessary if the machine tool ha an head/head configuration and is triggered by sending the tool tip data in the NC program. Therefore the controller has to calculate the values for the linear axis, if the rotary axis getting values. Set also the following figure and get an impression from the avi of a machine tool, which moves with RTCP (Retativ tool center programming). In later versions this data should be get out from the machine tool model (KIM) and is designed to describe all possible head/head configurations.

Example:	set mom_plugin_rtcp_data "4AXIS=<C> 5AXIS=<A> P2=<0.0 0.0 -8.30275590552> VN=<0.0 0.0 -1.0> M12=<1.0 0.0 0.0 0.0 1.0 0.0 0.0 0.0 1.0 0.0 0.0 -24.40944881890> MREF=<1.0 0.0 0.0 0.0 1.0 0.0 0.0 0.0 1.0 0.0 0.0 32.7122047244> AXIS1=<Z> AXIS2=<X> SIGN1=<+> SIGN2=<+>"
This example data are belong to the machine tool of the sim_sample_5x_achh_plugin.prt, which is used for demonstration purposes in IS&V.

The short video sequence shows the movement of a machine tool. The behaviour of three NC Blocks are shown. First positioning the tool tip onto the tip of the cone. Then moving the A Axis, the C axis then both simultaneously back to zero. This is shown first without RTCP and then with RTCP mode. It shows, that during RTCP mode also the linear axis has to do some movements, though the NC Block only triggers the rotary axis. To calculate these offsets of the linear axis some kinematic data of the machine tool has to be known.

Figure:	Video Sequence to show the behavior of a machine tool with RTCP mode

Figure:	Explain the mathematical Background and the data, which is needed to calculate RTCP

Mathematical Background:

The following formula describes the used mathematics in the generic PlugIn. The formula calculates the offsets of the linear axis as an vector by a given pair of axis values from the rotary axis. The other needed data is machine tool dependent and not dynamic.

tcp = mref *(m_rot_a1 *(m12 * (m_rot_a2 *(p2 + vn * len))))

m12	Transformation-Matrix from System K2 into System K1 (Rotation and Translation)

mref 	Transformation-Matrix from System K1 to the Reference-System (MTCS)

m_rot_a1	Rotation-Matrix about axis1 about the input value of the axis1 in system K1

m_rot_a2	Rotation-Matrix about axis2 about the input value of the axis2 in system K2

p2	the tool mount position in system K2

vn	the normal of the tool in system K2

len 	the length of the tool

Remarks: The axis2 is the dependent axis on axis1. This is the same for the Co-ordinate systems as shows in the figure above.

## Features of the generic example Plug-In.

This chapter listed all supported functions of the generic PlugIn. Furthermore the possible features are listed, which can be implemented after discussion about formats and syntax. For the possible features the principle functions, the math and the architecture are implemented.

### Miscellaneous Features and Code-words:

#### Supported Features:

- T – for selecting and changing the tool

- S - for setting the speed and issue feedback

- F – for setting the feed and issue feedback

- X,Y,Z,A,B,C – for setting axis values

- I,J,K – for center point of circular interpolation. A circle motion is triggered if the radius of the circle is greater than 0.01. A complete circle is assumed if the difference between the start and end position in radiant is smaller than 0.001

- Tool data description via variable setting (see settings in MTD)

- Supports up to 6 axis (3 linear, 3 angular) 
The key word syntax for the axis is only X,Y, Z, A, B, C

- ZCS-MTCS Offset calculation as described above.

- Tool length correction for the XY plane

- Circular interpolation – data for center point can given absolute or relative to the start point.

- Possible tolerance setting for the circular interpolation

- Initial settings: 
Work plane set to XY
Movement set to rapid mode
X,Y,Z values are given in absolute values

- Comments: all what is between () is ignored

- Issue extra function for a predefined start symbol. For % and # sign the offset between spindle junction and msys is created. Furthermore the junction ZSC is created and assigned to the work piece to allow display the tool values in the UI

#### Possible Features:

- Description of the syntax in a syntax.txt file
many different syntax description can be supported.

- Implement the control out functionality with a give “/” sign in front of each NC block

- Interpret R for radius description instead on in addition of center point for circular interpolation

- Parameter handling

- Position values (X,Y,Z) given in an arbitrary work plane.
For this feature definition data of the plane and the matrix are necessary

### G-Codes

#### Supported Features:

- G00 - for rapid motion

- G01 - for linear motion

- G02/G03 - for circular motions. Endpoint and center point required in NC block.

- G17/G18/G19 - for setting the work plane

- G53 – With this function all given linear motion values (X,Y,Z) are interpret relative to the MTCS.

- G90/G91 - to give the linear motion values (X,Y,Z) in absolute or incremental to the previous one.

#### Possible Features:

- G40 G41 G42 - Radius cutter compensation for some cases; This can only be supported if the IS&V architecture allows a look ahead functionality to scan the further motions. Otherwise this is not possible. At the moment the ISV not supports that, but this will be developed in the next time.

- G02/G03 - 3 dimensional circular interpolation in an arbitrary work plane

### M-Codes

#### Supported Features:

- M7/M8 – set the cooling mode on and issue feedback

- M9 – set the cooling mode off and issue feedback

### Possible Features:

- M6 – issue the tool change with the predefined tool by T